Table S1. Summary of selected articles and its key attributes.

| Authors and Year | Study Region | Methodology |  |  | Major Findings |
| --- | --- | --- | --- | --- | --- |
|  |  | Types of Sensors and Platforms | Spectral Characteristics of Water and Water Constituents | Applications for Different Water Bodies |  |
| (Ahmad et al., 2025) | Ethiopia | Landsat 8 satellite imagery | Turbidity | Freshwater lake | Among the three regression models tested OLS polynomial regression proved the most effective in modeling turbidity distribution based on spectral reflectance values. |
| (Muhoyi et al., 2025) | Zimbabwe | Cloud free Sentinel-2 satellite image data | COD, TN, BOD5, TP, and TSS | River | The remote sensing techniques’ results showed that remote sensing-based empirical algorithms could be used in the monitoring and protection of remote catchments such as LMS. |
| (Kandasamy et al., 2025) | India | Landsat-8 (Coastal/Aerosol, Blue, Green, and Red) | Chl-a concentrations in aquatic settings | River | Introduces a new hybrid model that was synergized with established algorithms by merging deep learning innovations with a flexible approach to water quality analysis for global applications. |
| (Dawn et al., 2025) | India | Landsat-8 satellite imagery | turbidity, TDS, and BOD | Lake | Regression achieving the highest accuracy for BOD prediction (R2 = 0.99). Spatiotemporal patterns revealed persistently high BOD levels along urban lake fringes and post-monsoon spikes in turbidity and TDS, especially in agriculturally influenced zones. |
| (Tavares et al., 2025) | France | Sentinel-2 MSI | Chl-a and turbidity | Lake | The temporal and spatial representations of the selected parameters were also shown to be consistent, demonstrating that the framework is robust and can be applied over lakes as small as 3 ha. |
| (Choto et al., 2025) | Japan | GCOM-C/SGLI satellite imagery | absorption of particles, phytoplankton, non-algal particles, CDOM, Chl-a | Eutrophic Lake | IOPs as variables for estimating water quality parameters such as Chl-a and suspended solids showed better performance compared to empirical methods |
| (Joshi et al., 2025) | North America | Harmonized Landsat Sentinel-2 (HLS) images | Chl-a | Lake | XGB provided the most stable and consistently strong performance across input configurations, which could be an important consideration for operational use and the findings are highly relevant for harmful algal bloom (HAB) monitoring |
| (Kokolakis et al., 2025) | Greece | RGB sensors with a multispectral camera, drone imagery, UAV data | Normalized Difference Chlorophyll Index (NDCI), Normalized Difference Algae Index (NDAI), Maximum Chlorophyll Index (MCI), and Cyanobacteria Index (CI) | Stream | Demonstrate the effectiveness of combining RS techniques, including satellite and drone imagery, with in situ measurements such as Spectral Induced Polarization (SIP), GEM-2, and chemical analyses. |
| (Kong et al., 2025) | United States | Sentinel-2A/B and Landsat 8-9 (L8-9) | Turbidity | Coastal water | Satellite data were able to estimate in situ turbidity, especially when enhanced with machine learning algorithms. Among the studied machine learning models, RF provided the best performance, highlighting its robustness in capturing the relationship between satellite-derived and in situ turbidity. |
| (Salem et al., 2025) | Japan | Global Change Observation Mission-Climate (GCOM-C) carrying the Second-generation Global Imager (SGLI), G-Portal and JASMES | Chl-a and total suspended matter | Ocean | G-Portal performed better at shorter wavelengths, showing lower errors and better correlation. |
| (Synan et al., 2025) | USA | Landsat 8 (Operational Land Imager) platform | Chl-a | Coastal water | Predictions of water quality indices from both C2RCC and random forest techniques can be a useful addition to existing water quality monitoring efforts, potentially expanding both spatial and temporal coverage of monitoring efforts. |
| (Wentao Hu & Jin, 2025) | China | GF-1, Landsat-8, and Sentinel-2 data | TN and TP | River | The three-satellite GA-RF joint model shows the best estimation performance from the in-situ measurements |
| (X. Chen et al., 2025) | China | Sentinel-2 | Total suspended matter (TSM) concentration | Lake | The DFE-ML framework also improved the discriminative power of spectral features, overcoming the limitations of manual feature selection and traditional statistical regression methods |
| (Assaf et al., 2025) | Jordan | Sentinel-2 MultiSpectral Instrument (MSI) | Chl-a | Dam | Among the models, RF demonstrated the highest predictive accuracy (R² = 0.93 calibration, R² = 0.91 validation), outperforming CB and XGB, whereas linear models such as RR and SVR were inadequate for capturing the nonlinear spectral relationships of Chl-a. |
| (Barkhordar et al., 2025) | Iran | Sentinel-2A satellite Sentinel-1 Level 3 Sentinel 3A-OLCI | Chl-a concentration | Gulf | Through the integration of high-resolution Sentinel-2 and Sentinel- 1 satellite data alongside in-situ radiometric measurements, a robust model has been developed that effectively captures both spatial and temporal variations in Chl-a concentrations. |
| (Huan Li et al., 2025) | Hungary | Landsat 4–9 | Chl-a | Lake | The result also shows a reasonable trade-off via increasing the number of matchups 3 to 8 times by extending the time tolerance from the same day to 3 days regardless of the high variability of Chl-a due to the sinking and floating movement of algae |
| (Batina & Siljeg, 2025) | Croatia | Geographic Information System-based Multi-Criteria Decision Analysis (GIS-MCDA) Integrated with the Fuzzy Analytical Hierarchy Process (F-AHP) | Seasonal water level, fluctuations, salinization from the Adriatic Sea, and various anthropogenic and environmental pressures | Coastal shallow lake | One of the most significant findings was the identification of spatial heterogeneity in water quality, with lower quality observed in the northwestern and southern parts of the lake |
| (Mussina et al., 2025) | Kazakhstan | Sentinel-2(A and B), Landsat-8 Operational Land Imager (OLI), Landsat-7 Enhanced Thematic Mapper (ETM+), and Landsat-5 Thematic Mapper (TM), ArcGIS 10.8, ArcGIS Pro | DO, TSS, and TDS | River | Confirmed the effectiveness of integrating remote sensing and geographic information systems to analyze spatial–temporal patterns in river system dynamics |
| (Batina & Siljeg, 2025) | Croatia | PlanetScope satellite imagery Geographic Information System-based Multi-Criteria Decision Analysis (GIS-MCDA) integrated with the Fuzzy Analytical Hierarchy Process (F-AHP) | Eutrophication | Lake | One of the most significant findings was the identification of spatial heterogeneity in water quality, with lower quality observed in the northwestern and southern parts of the lake |
| (Tcherkezova et al., 2025) | Bulgaria | Sentinel-2 Level 1C, QGIS plugin “Thresholds to ROI” | WQI | River | The integrated GIS-based analysis allowed us to determine the extent of water bodies in different years and seasons, the degree of surface water pollution, and the points for long-term monitoring of river water quality in the investigated area |
| (J. Chen et al., 2025) | China | PlanetScope images | Chl-a, DO, COD, and PO4-P | Marine | SVR demonstrated optimal performance in the inversion of all water quality parameters. This effectiveness is attributed to the fact that marine water quality parameters are significantly influenced by nonlinear processes, which are intertwined with physical, biological, and chemical factors |
| (Zhou et al., 2025) | China | Sentinel-2 multispectral imagery | TP | Lake | The findings provide a significant reference for remote sensing monitoring of lake water quality and can be used to predict and regulate salinity, eutrophication, and similar conditions in comparable lakes |
| (M. Kiwanuka et al., 2025) | Uganda | Landsat 8/9 imagery | Chl-a and SDD | Lake | Results revealed high algal concentrations for nearshore and clearer offshore waters. These models provide an efficient framework for monitoring eutrophication, guiding restoration priorities, and supporting sustainable water management in Lake Victoria |
| (Luo et al., 2025) | China | Hyperspectral Proximal Sensing system (HPSs) | lake surface water temperature (LSWT) | Lake | The study provides a practical early warning and management tool to mitigate harmful algal blooms and safeguard drinking water security under climate change |
| (Igoe, 2025) | Ireland | Landsat-8 and Sentinel-2 imagery | Suspended Sediment Concentration (SSC) | Coastal water | The model effectively captured key spatio-temporal patterns of relative SSC in shallow coastal waters, demonstrating strong performance across multiple scales |
| (Al et al., 2025) | Bangladesh | Sentinel-2 surface reflectance images | TDS, TSS, and electrical conductivity (EC) | River | The findings highlight the significant influence of seasonal flooding and anthropogenic discharges on water quality degradation, posing a threat to fish health and public safety |
| (Kharabsheh & Bdour, 2025) | Jordan | Real-Time Remote monitoring (RTRM) | dissolved oxygen (DO), pH, conductivity, turbidity, and sediment concentration, | Marine water | Results showed that DO levels generally supported aquatic life, though northern coastal areas were more vulnerable due to localized pollution. Turbidity and sediment patterns highlighted recreational disturbances, particularly from boating |
| (Moses Kiwanuka et al., 2025) | Uganda | Landsat 8 and 9 imageries | Chl-a, turbidity, and Trophic State Index (TSI) for eutrophication | Lake | The study demonstrates satellite remote sensing as an efficient tool for monitoring water quality changes, guiding restoration measures, and identifying highly contaminated zones through threshold-based segmentation of satellite-derived water quality maps, rather than assessing the lake as a uniform whole |
| (Yan et al., 2025) | China | OLCI/Sentinel-3 satellite imagery | Algal biomass abundance | Lakes and reservoir | The findings suggest that temperature, wind speed, and atmospheric pressure are critical factors influencing the biomass abundance of the different algae groups |
| (Dawn et al., 2025) | India | Copernicus Land Monitoring Service or CLMS, European Space Agency Lakes Climate Change Initiative | surface water temperature (LSWT), turbidity, and Chl-a | Lake | Validation of remote sensing derived data showed that LSWT estimates are better correlated with in situ data than turbidity estimates |
| (Rodríguez-lópez et al., 2025) | Chile | Landsat 8 and 9 satellite imagery | harmful algal blooms (HABs) | Lake | These ﬁndings offer a useful foundation for developing early warning systems to support lake management strategies, especially in regions facing increasing pressure from climate change and eutrophication |
| (Steinbach et al., 2025) | Kenya | Sentinel-2 Level-1C (1C) data from the multispectral instrument (MSI) | Turbidity | Small reservoir | The results suggest that short- and longer-term turbidity prediction can inform reservoir siting and management |
| (Hongran Li et al., 2025) | China | Unmanned Aerial Vehicles (UAVs) | TN, DO, TSS, and Chl-a | Urban river | A spatial analysis reveals significant heterogeneity in water quality distribution across the study area, with natural water bodies exhibiting relatively uniform conditions, while the concentrations of TN and TSS are substantially elevated in aquaculture areas due to aquaculture activities |
| (Adilakshmi & Venkatesan, 2024) | India | Landsat 8/9 OLI/TIRS (Operational Land Imager/ Thermal Infrared Sensor) | temperature, conductivity, TDS, pH, turbidity, Chl-a, | River | The water quality index consistently indicated low water quality and the prediction models employing the LASSO approach yield promising results, with R-squared values exceeding 0.87 for temperature and pH |
| (Zhao et al., 2024) | China | Sentinel-2 satellite images | Permanganate Index, TN, TP, ammonia nitrogen, and water turbidity | Inland waters | Comparisons between satellite-retrieved and in situ values showed good consistency in both spatial and temporal distributions |
| (Ali et al., 2024) | China | Sentinel-2 | Chl-a, SDD, and TSS | Lake | The DNN models outperformed in effective evaluation of Chl-a, SDD on test datasets and Chl-a, TSS and SDD against Sentinel-2A validation dataset of Liangzi lake |
| (Z. Li et al., 2024) | China | Satellites Sentinel-2A and Sentinel-2B | Chromophoric Dissolved Organic Matter (CDOM) | Lake | The findings offer hints for an accurate evaluation of watercolor parameters of Ebinur Lake and practical references for monitoring arid-region lake water quality via remote sensing |
| (Jiang et al., 2024) | Iraq | TOPEX/Poseidon, ERS-2, Envisat, Cryosat-2 and Sentinel-3 Ocean and Land Colour Instrument | Chl-a concentration and turbidity | Lake | Chl-a increase in the lake was mainly explained by built-up area increase and water area decrease in the catchment, with a relative contribution of 29.2 % and 28.6 % respectively. |
| (Eljaiek-Urzola et al., 2024) | Columbia | Sentinel-2 images | Turbidity and SDD | Bay | The results indicate that the algorithm based on the red band had a better performance for retrieving water turbidity, with a good validation accuracy of R2 = 0.83 |
| (P. Li et al., 2024) | China | Landsat 8 remote sensing images | Eutrophication levels | Shallow freshwater lake | RF is the most appropriate machine learning model to be used directly to reflect the TLI and indicated a simulation accuracy that far exceeds the level of other models. |
| (Rivera-Ruiz et al., 2024) | Chile | Multispectral Instrument (MSI) aboard Sentinel-2A/B | Secchi depth | Eutrophic lake | C2XC outperforms in higher trophic levels, while challenges persist in accurately assessing lower trophic state categories |
| (Sang et al., 2024) | China | Shipborne LiDAR data | Chl-a | Inland water | The findings of this study suggest that lidar technology has significant potential for the long-term monitoring of inland lakes |
| (Sun et al., 2024) | China | Sentinel-2 Data | TP concentration | Freshwater lake | The results indicated that the bagging model, by integrating multiple different models, mitigates the prediction bias of individual models, resulting in higher inversion accuracy and smaller errors compared to the other three methods |
| (Schmidt et al., 2024) | Germany | Sentinel-2 MSI and Sentinel-3 OLCI | Chl-a, Secchi depth, and turbidity | Inland water | Remote sensing may, thus, be regarded as a valuable tool for complementing in situ monitoring by useful information on higher spatial and temporal scales to support water management. |
| (Joshi et al., 2024) | North America | Sentinel-3 OLCI (Ocean and Land Color Imager) images | Chl-a, Microcystin, Phycocyanin, and Secchi-depth | Lake | Microcystin, responsible for bloom toxicity, has limited spectral sensitivity, and Secchi depth could be influenced by various factors besides blooms, such as colored dissolved organic and inorganic matter. |
| (Lehua Dong et al., 2024) | China | Landsat 8/9 imagery and sea surface temperature (SST) | DO, Chl-a, total suspended matter (TSM) concentrations, and thermal effluents | Coastal water | The research results indicate significant seasonal variations in the DO concentration, with higher concentrations in winter and lower concentrations in summer. Additionally, DO concentrations in coastal areas were generally greater than those in outer continental shelf waters. |
| (Z. Zhang et al., 2024) | HongKong | MYDOCGA Version 6 Level 2 Gridded Lite (L2G-lite) Ocean Reflectance product (2002–2023) derived from MODIS. | TN and TP | Coastal water | The results suggest TN and TP concentrations are highest in Shenzhen Bay, and this region’s concentrations were declining during 2009–2019 |
| (Coccia et al., 2024) | Spain | Hyperspectral images acquired by the CASI-1500 hyperspectral airborne sensor | water depth, turbidity and Chl-a | Pond | The high spatial and spectral resolution of the CASI sensor allowed to detect differences between ponds in water depth, turbidity and Chl-a, providing accurate mapping of these three variables, and a useful method to assess restoration success. |
| (Lei Dong et al., 2024) | China | Airborne multispectral and multimodal imaging spectrometer (AMMIS) | TN and TP concentrations | Lake | Spectral reflectance bands exhibit greater sensitivity to TN and TP concentrations compared to the winter and spring seasons. Areas with intense algal blooms near shorelines generally exhibit higher TN and TP concentrations. |
| (Bygate & Ahmed, 2024) | Texas | Landsat-8 (Collection 2, Level 2, Tier 1) | Chl-a, salinity, and turbidity, | Bay | ML models outperformed calibrated empirical models when used to retrieve turbidity and salinity, DNN family outperformed all other ML families when used to retrieve salinity |
| (Jakovljevic et al., 2024) | Serbia | Landsat 5, Landsat 7, and Landsat 8 surface reflectance products | TSS, Chl-a, (DO), TN, and TP | River | The ANN-based model exhibited very high accuracy, particularly for Chl-a and TSS. The independent validation showcased accurate classification for TSS and TN, while DO estimation faced challenges during high variation periods, highlighting the complexity of DO dynamics |
| (Anand et al., 2024) | India | Sentinel-2 and ResourceSat-2 [Linear Imaging Self-Scanning Sensor (LISS)–IV] multi-spectral satellite data, | pH, DO, TSS and TDS | Lake | The result of this study indicates that the SVM yielded the highest accuracy followed by the RF and MLR. |
| (Zhong et al., 2024) | China | Moderate Resolution Imaging Spectroradiometer (MODIS) on the Aqua satellite, | Sea Surface Nitrate (SSN) concentrations | Seawater | The SSN at the global scale and in the North Pacific Ocean show a decreasing trend, a phenomenon that this study demonstrates to be related to global warming, and factors associated with climatic conditions. The SSN and Chl-a in the eastern equatorial Pacific respond more strongly to strong ENSO events |
| (Rao et al., 2024) | China | Sentinel-2 satellite data | Chl-a | Shallow lakes | Underscores the potential of integrating machine learning models with Sentinel-2 data to enhance the monitoring and prediction of phytoplankton dynamics in shallow lakes. This approach offers valuable insights into the early detection of algal blooms and informed management strategies, contributing to the preservation and sustainable management of aquatic ecosystems |
| (Min et al., 2024) | Mary Land, USA | Drone-based RGB imagery data | Escherichia coli concentrations | Irrigation pond | The combination of water quality and RGB imagery data resulted in a higher R2 for the test dataset. |
| (Lei et al., 2024) | China | Unmanned Aerial Vehicle (UAV) and multispectral remote sensing | Optically active parameters (OAPs, including Chl-a and SD) and non-optically active parameters (including CODMn, TN, and TP) | Urban reservoir | The results showed that the upstream head and downstream tail of the reservoir suffered from nitrogen–phosphorus nutrient and nitrogen nutrient pollution. TP, TN, and Chl-a were lower in the dry season than in the rainy season, and clarity and CODMn were higher in the dry season |
| (Islam et al., 2024) | Bangladesh | Combined multivariate statistical analysis and geographic information systems. deterministic inverse distance weighted (IDW) method, ArcGIS 10.5 | Physicochemical properties | River | This enhanced IWQI offers a promising tool for seasonally adaptive water management, promoting sustainable agriculture and soil health amidst climate change. Future research should expand the temporal and spatial scope to capture long-term trends and adapt this |
| (Ramachandran et al., 2024) | India | Sentinel-2A | pH, Electrical Conductivity (EC), TDS, DO, Calcium (Ca), Magnesium (Mg), Total hardness, Chloride (Cl-), Carbonate, bicarbonate | Lake | Correlation and regression techniques have been employed to integrate these datasets, resulting in the development of a robust predictive model. By combining these two information sources, the model is constructed using stepwise regression analysis |
| (Hasan & An, 2024) | Korea | Sentinel-2 Multi-Spectral Imager (MSI) and Landsat-8 Operational Land Imager (OLI) satellite imagery | Chl-a, SDD, and TSS | Reservoir | This approach has the potential to yield valuable insights aiding the assessment and management of water quality at the regional, national, and global levels |
| (Meng et al., 2024) | China | Sentinel-2 images | Area of algal bloom (AAB) | Lake | The findings revealed an improvement in Lake Dianchi's water quality, with Levels A (good water quality) and B (mildly polluted water quality) averaging 1.24% and 84.28%, respectively |
| (Cao et al., 2023) | China | Sentinel-2 image | DO, permanganate index, ammonia nitrogen, TP, TN, and turbidity | River | The inversion accuracies of the five inversion models indicated the feasibility of the water quality inversion method based on Sentinel-2 data using statistical theory for monitoring water quality concentration in the YRD. |
| (Ahmed et al., 2023) | Iraq | Landsat 8 images | temperature, electrical conductivity, TDS, pH, turbidity, Chl-a, blue-green algae, and DO | River | The predictive model for each parameter provided cost-effective alternatives to frequent monitoring of Tigris water quality using field data. The predicted parameters were then utilized to calculate the water quality index (WQI) to indicate water quality along the river. |
| (Kowe et al., 2023) | Zimbabwe | cloud free and remotely sensed Sentinel 2 satellite data | Total Nitrogen (TN), Turbidity, Chl-a (Chl-a) and Total Suspended Matter (TSS) | Freshwater in inland water bodies | The application of fine spatial resolution data like the recently launched Sentinel 2 satellite imagery and statistical models are useful for future monitoring water quality parameters, eutrophication and water pollution trends in inland water bodies such as Lake Manyame. |
| (Assegide et al., 2023) | Ethiopia | Sentinel-2 satellite remote sensing data | Chl-a, turbidity, and TSS measurements | Reservoir | Sentinel 2 derived regression models can support the spatiotemporal estimation and mapping of the annual and monthly patterns of Chl-a, TU, and TSS over the Koka reservoir. |
| (F. Zhang et al., 2023) | China | Landsat 8 OLI and ESTARFM | Suspended particulate matter | Brackish water (lake) | The overall results of the blue, green, red, and NIR bands generated by ESTARFM were better than FSDAF when compared with the real images. |
| (Pelevin et al., 2023) | Kyrgyzstan | Laser-induced ﬂuorescence (LIF) by a marine LIDAR in situ and Sentinel-2/MSI image | Chl-a, CDOM, and TSS | Lake | The development of regional bio-optical models and algorithms for interpreting satellite data on the watercolor has good applicability not only for eutrophic waters but also for oligotrophic waters |
| (Jin et al., 2023) | China | Landsat5 TM, Landsat7 ETM+, Landsat8 OLI | SDD, Chl-a, and suspended solids (SS) concentration | River | The water body extraction accuracy of the EGS operational surface water mapping algorithm, combined with Landsat data, is superior |
| (Lei Dong et al., 2023) | China | Sentinel-2 MSI images | Permanganate index (CODMn), Tn and TP Chl-a, DO, and SDD | Lake | Permanganate index (CODMn), Total Nitrogen (TN), and Total Phosphorus (TP) shows relatively small interannual differences, generally exhibiting a decreasing trend in concentrations. In contrast, Chl-a, DO, and SDD exhibit signiﬁcant interannual and inter-year differences |
| (Maniyar et al., 2023) | Central America | Sentinel-2 reﬂectance data | TSS | Coastal water | DNN performed the best and estimated TSS with a testing R2 of 0.89. |
| (Villota-González et al., 2023) | Mexico | Landsat-8 and Sentinel-2 images | Chl-a, turbidity, and TSS | Lake | Better predictive performance was observed for turbidity with Landsat-8 radiometric data, while TSS improved when Sentinel-2 data were employed |
| (Qiang et al., 2023) | Russia | Sentinel3-OLCI sensor | Chromophoric dissolved organic matter (CDOM) | Lake | The signiﬁcant relationship between CDOM and DOC (HIX and chemical oxygen demand (COD)) indicated the potential remote sensing application of water quality monitoring for water management |
| (Patricio-Valerio et al., 2023) | Australia | TSS | Total Suspended Solids (TSS) | River | Himawari-8 TSS products compared remarkably well to concurrent measurements of in situ TSS derived from particulate backscattering measurements at the LJCO. |
| (Najafzadeh & Basirian, 2023) | USA | Landsat 8 OLI-TIRS images | Turbidity, Sulfate, Sodium, Potassium, Hardness, Fluoride, Dissolved Oxygen, Chloride, Arsenic, Alkalinity, pH, Nitrate, and Magnesium | River | The results of the AI models’ performance showed that the MARS model had the best performance among the other AI models for monitoring WQI. |
| (Wang et al., 2023) | HongKong | 117 Landsat 5 TM and 44 Landsat 8 OLI images | concentrations of suspended particulate matter, orthophosphate phosphorus, and dissolved inorganic | Marine | Proves that Landsat is much more applicable compared with MODIS and HY-1C CZI when exploring the impact of regional long-term human activities |
| (Sedighkia et al., 2023) | Iran | Landsat 8 | TDS | Lake | Based on the results in the case study, the monitoring model was sufficiently robust to convert the operational land imager spectral bands of Landsat 8 to the TDS distribution map |
| (Rodríguez-López, Usta, et al., 2023) | Chile | Landsat-8 (L-8 OLI) images | Chl-a concentration | Lake | The three estimation models employed demonstrated strong performance in estimating Chl-a, with the LSTM model yielding the most accurate results. |
| (Arias-Rodriguez et al., 2023) | USA, Europe, Mexico, Canada, Japan, Chile, Russia | Landsat-8 and Sentinel-2 | SDD, TURB and BOD | Coastal surface water | The results show that trained models achieve moderately high correlations for SDD, TURB and BOD but lower trained models achieve moderately high correlations for SDD, TURB and BOD but lower performances for TSM and NO3-N |
| (Rodríguez-López, Duran-Llacer, et al., 2023) | Chile | Landsat 8 Collection 2 Level 1 Operational Land Imager images (L8/OLI) | Chl-a, TN, and TP | Lake | The statistical indicators of the model generated revealed that the SABI and FAI indices were the most precise in estimating Chl-a levels in Lake Villarrica. |
| (X. Zhang et al., 2023) | China | Landsat and Moderate Resolution Imaging Spectroradiometer (MODIS) data | Total suspended solids (TSS) concentration | Bay | Both Landsat and MODIS retrieval models proposed in this study have good accuracies |
| (J. Xu et al., 2023) | China | Sentinel-2 A/B Multi-Spectral Instrument (MSI) observations (Level 1C) | Chl-a | Lake | The simulation results reveal that the flow speed is a significant factor modulating the lake Chl-a with a high concentration in summer and a low concentration in winter; the flow speed and water storage exceed the temperature and solar radiance in modulating the Chl-a |
| (Xiao et al., 2023) | China | Sentinel-2 MultiSpectral Instrument (MSI), Landsat-8 Operational Land Imager (OLI) and Landsat-9 OLI2 data | Chl-a and suspended sediment (SS) concentrations | River | The typical Chl-a retrieval algorithm, NDCI, is only effective in certain bands. It is also noted that Landsat8 and Landsat9 lack some key band settings (e.g., the red-edge band of 700–710 nm), severely limiting practical application in relation to Chl-a |
| (Gupta et al., 2023) | North America | MERIS and Sentinel-3 OLCI satellite data | Harmful algal blooms of cyanobacteria (CyanoHAB) | Lake | The best predictions were obtained by using the RF and EA algorithms. It was found that CyanoHAB growth dynamics, even at sub-monthly timescales, are determined by coarser timescale variables |
| (Fickas et al., 2023) | United States | Sentinel-2 (S2) MultiSpectral Instrument (MSI) and the Sentinel-3 (S3) Ocean Land Colour Instrument (OLCI) | Cyanobacteria harmful algal blooms (cyanoHABs) | Lake | Comprehensive analyses indicate increased spatiotemporal coherence of bloom magnitude, frequency, occurrence, and extent of MDN-based maps compared to CI-cyano and potential for use in cyanoHAB monitoring for public health and aquatic resource managers |
| (Alharbi, 2023) | Saudi Arabia | MODIS/Aqua satellite data, Landsat-8/OLI satellite data | Chl-a; Concentration and Sea Surface Temperature (SST); Normalized Difference Turbidity Index (NDTI); Surface Scum Index (SSI); Surface Algal Bloom Index (SABI); Normalized Difference Vegetation Index (NDVI), and Normalized Difference Water Index (NDWI) | Sea coast | The study produced a digital map of algal blooms in the study area. Algal blooms exhibited medium inverse correlations with temperature and specific humidity, and strong direct correlation with wind speed |
| (Wentong Hu et al., 2023) | China | Unmanned Aerial Vehicles (UAVs) | TP Concentration | Small inland waterbodies | The extreme gradient boosting optimized by genetic algorithm (GA-XGB) performed best, with the highest precision for the validation and test sets |
| (Werther et al., 2022) | Scotland, Estonia, New Zealand, Switzerland | Sentinel-3 Ocean and Land Cover Instrument (OLCI) and Sentinel-2 MultiSpectral Imager (MSI). | Chl-a concentration | Oligotrophic and mesotrophic lake | This study serves as research on Bayesian machine learning for the estimation and visualization of Chl-a and associated retrieval uncertainty to develop harmonized products across OLCI and MSI for small and large oligo- and mesotrophic lakes |
| (Júnior et al., 2022) | Brazil | Sentinel-3 OLCI | Chl-a concentration | Lake | The results showed a smaller mean absolute percentage error (MAPE) for HSAA Chl-a retrieval (36.93%) than empirical models (73.39%) using a 3-band algorithm, which conﬁrms the better performance of the semi-analytical approach |
| (Fu et al., 2022) | China | Landsat 8 OLI(L8), Sentinel-2 A/B(S2) images | Chl-a, Total suspended matter (TSM) and NOAPs (total phosphorus (TP), total nitrogen (TN), ammonia nitrogen (NH4-N), five-day biochemical oxygen demand (BOD5)) | Lake | Estimating accuracy of non-optically active parameters was higher than optically active parameters, but the accuracy differences of non-optically active parameters were greater than optically active parameters |
| (Caballero et al., 2022) | Spain | Sentinel-2A/B twin satellites and Landsat-8 satellite | Chl-a concentration | Seawater | The present multi-sensor strategy is an excellent opportunity to highlight the potential of remote sensing technology as a relevant and powerful tool for future hazard monitoring and assessment during catastrophes and for a better interpretation of their impact on the marine environment |
| (E. Kim et al., 2022) | Korea | Unmanned Surface Vehicles (USVs) | Chl-a concentrations | Stream | The spatial distribution map exhibited considerable potential for the commercialization of remote monitoring technology development using USVs |
| (C. Li et al., 2022) | Switzerland, France | Sentinel-2 and Sentinel-3 satellites | Total Suspended Matter (TSM) | Lake | The results demonstrate that remote sensing images can serve to calibrate and validate particle tracking models with independent observations |
| (H. Zhang et al., 2021) | China | MODIS/Aqua Level-1A dat | Chl-a and DOC | Lake | This study provides a feasible remote sensing method that developed algorithms for studying the temporal and spatial dynamics of the CDOM concentration and fluorescence indices in Erhai Lake, which is helpful to obtain the CDOM concentration and sources information |
| (H. Xu et al., 2021) | China | Landsat-5 Thematic Mapper (TM), Landsat-8 Operational Land Imager (OLI) | TSS concentrations | River | The results show that the recalibrated algorithm is highly consistent with the two algorithms. All of the three algorithms indicate significant fluctuation in TSS concentrations in the Lower Min River during the study period. |
| (Binding et al., 2021) | Canada | Sentinel-3A satellite MERIS Level-1 FR imagery | Algal bloom | Lake | Results document widespread blooms on each lake, with maximum spatial extent of 21,641 km2 (representing 88.1% of the lake area) on LW, 3070 km2 (79.5%) on LoW and 5257 km2 (19.7%) on LE |
| (Qing et al., 2021) | China | Sentinel-2 Multispectral Instrument (MSI) | Secchi disk depth (SSD) | Lake | The results showed that ZSD retrieval in optically complex waters can be significantly improved by refining the empirical equation for a (λ0) and blending optimal models for various water types according to optical classification |
| (Y. H. Kim et al., 2020) | Korea | Moderate Resolution Imaging Spectroradiometer (MODIS) and Visible Infrared Imaging Radiometer Suite (VIIRS) products | DO, sea surface temperature (SST), and Chl-a | Coastal water | This shows the potential capability of satellite remote sensing in monitoring in situ DO in both high temporal and spatial resolution, which will be beneﬁcial for eﬀective and eﬃcient management of coastal environments |
| (Keith, 2014) | North Carolina | Multispectral Scanner (MSS) data from a Landsat-3 | Chl-a, TN | Estuaries | The MERIS Chl-a data product was successfully used to conduct a retrospective analysis of the chlorophyll-based TMDL for total nitrogen which serves as the cornerstone of the nutrient management strategy for the Neuse and Pamlico River estuaries |

Table S2. Risk of Bias Assessment of Included Studies

| Bias Domain | Assessment Criteria | Common Issues Identified in Reviewed Studies | Overall Risk Level |
| --- | --- | --- | --- |
| Data Quality | Use of high-quality satellite data and sufficient in-situ measurements for calibration and validation | Limited or uneven in-situ data; reliance on sparse field samples; inconsistent temporal alignment between satellite and ground data | Moderate |
| Validation Strategy | Use of independent validation datasets, spatial–temporal validation, and robust cross-validation techniques | Over-reliance on random cross-validation; lack of independent datasets; small sample sizes leading to overestimated model accuracy | High |
| Model Transparency & Reproducibility | Clear reporting of methods, algorithms, and parameter settings; reproducibility of workflows | Incomplete methodological descriptions; lack of open data/code; unclear model configurations in some studies | Moderate |
| Confounding Factors Handling | Proper treatment of atmospheric correction, adjacency effects, and optical complexity of water bodies | Insufficient atmospheric correction; limited discussion of adjacency effects; unaddressed optical variability in complex waters | High |
| Generalizability | Applicability of models across different regions, seasons, and water types | Strong site-specific calibration; poor transferability across regions and time; lack of external validation | High |

Table S3. Summary of categorized studied regions

| Country / Region | Classification* |
| --- | --- |
| Australia | Developed |
| Austria | Developed |
| Bangladesh | Developing |
| Brazil | Developing |
| Bulgaria | Developed |
| Canada | Developed |
| Central America | Developing |
| Chile | Developing |
| China | Developing |
| Colombia | Developing |
| Croatia | Developed |
| Estonia | Developed |
| Ethiopia | Developing |
| France | Developed |
| Germany | Developed |
| Greece | Developed |
| Hong Kong | Developed |
| Hungary | Developed |
| India | Developing |
| Iran | Developing |
| Iraq | Developing |
| Ireland | Developed |
| Japan | Developed |
| Jordan | Developing |
| Kazakhstan | Developing |
| Kenya | Developing |
| Kyrgyzstan | Developing |
| Mexico | Developing |
| New Zealand | Developed |
| North America | Developed |
| Russia | Developing |
| Saudi Arabia | Developing |
| Scotland (UK) | Developed |
| Serbia | Developing |
| South Korea | Developed |
| Spain | Developed |
| Switzerland | Developed |
| Uganda | Developing |
| United States | Developed |
| Zimbabwe | Developing |

* Countries were classified as developed or developing based on the World Bank income classification. High-income economies were

categorized as developed, while low- and middle-income economies (including upper-middle-income countries) were categorized as

developing.

Figure S1. Regional distribution of selected studies